3-Part CSI MasterFormat Specification

SECTION 05 5000

Stair Treads and nosings

Rev 10/24

GENERAL

      - SUMMARY

        - Section includes:

Anti-slip Safety Stair Treads and Nosing’s for new construction and renovation, inside and outside stair applications including extruded aluminum and cast metal systems.

      - references

        - State of California Title 24, Striping code for the visually impaired

        - American Disability Act (ADA) barrier-free code design

        - Occupational Safety and Health Association (OSHA)

        - International Building Code (IBC)

        - Underwriters Laboratories (UL) 1994 Luminous Egress Path Markings

        - ASTM B 221 Standard Specification for Aluminum and Aluminum Alloy extruded bars, rods, wire, shapes and tubes.

        - ASTM 2072 Standard Specification for Photoluminescent (Phosphorescent) Safety Markings.

      - coordination

        - Coordinate with other trades such as concrete and masonry.

      - Action SUBMITTALS

        - Shop Drawings: Indicate configuration and dimensions, show components, adjacent construction, required clearances and tolerance and other affected work.

        - Product Data:  Manufacturer’s technical data for each type of floor door, including setting drawings and finish requirements

      - informational submittals

        - Provide manufacturer's standard warranty.

        - Sustainable Design Submittals:

Building Product Disclosure Requirements: To encourage the use of building products that are working to minimize their environmental and health impacts, provide the following information when available:

Material Ingredients Documentation demonstrating the chemical inventory of the product

      - Closeout submittals

        - Manufacturer's Installation Instructions and Operation & Maintenance:  Indicate installation, operation and maintenance requirements and rough-in dimensions.

PRODUCTS

      - MANUFACTURER

        - Nystrom

9300 73rd Ave N

Brooklyn Park, MN  55428

PH: 	(800) 547-2635

www.Nystrom.com

      - abrasive metal [NOSINGS] [TREADS] [AND] [THRESHOLDS]

Retain product and nosing types required and revise to suit Project. Delete all if configurations are indicated on Drawings.

        - Single Part Unit: Aluminum with abrasive filler consisting of aluminum oxide, silicon carbide, or a combination of both, in an epoxy-resin binder.

Ribbed bar abrasive.  Aluminum alternating with abrasive filler consisting of aluminum oxide, silicon carbide, or a combination of both, in an epoxy-resin binder

Basis of-Design Product: Nystrom, STSB

Nosing Types:

Short Nose Tread: [1-7/8 inches (48 mm)] [3 inches (76 mm)] [4 inches (102 mm)] wide, with 1/2 inch (13 mm) lip for poured concrete applications.

                - Attachment Method: [Cast-In, continuous extruded anchor] [Drilled Hole, Mechanically Fastened].

Short Tipped Nose: [1-7/8 inches (48 mm)] [3 inches (76 mm)] [4 inches (102 mm)] wide, with 1/4 inch (6 mm) lip for poured concrete and steel pan applications.

                - Attachment Method: [Cast-In, continuous extruded anchor] [Drilled Hole, Mechanically Fastened].

Angled Long Nose Tread: [1-7/8 inches (48 mm)] [3 inches (76 mm)] wide, with 1-1/8 inch (31 mm) lip, 83 degree angle for sloped risers.

                - Attachment Method: [Cast-In, continuous extruded anchor] [Drilled Hole, Mechanically Fastened].

Inset, Square Front and Back: [1-3/8 inches (35 mm)] [3 inches (76 mm)].

                - Attachment Method: [Cast-In, continuous extruded anchor] [Drilled Hole, Mechanically Fastened].

Finish: 6063-T5 aluminum, mill finish.

Abrasive Colors: [Black] [Safety yellow] [see manufacturer for available color selections].

Specify taped top or stair nosing guard to prevent concrete from penetrating tread during installation. Painted undersides create a barrier between metal and concrete to avoid adverse reaction.

Options:

Taped Top.

Painted Undersides.

Full abrasive filler.  Aluminum filled with abrasive consisting of aluminum oxide, silicon carbide, or a combination of both, in an epoxy-resin binder and aluminum extrusion.

Basis-of-Design Product: Nystrom; STSF

Nosing Types:

Short Nose Tread: [2 inches (51 mm)] [3 inches (76 mm)] [4 inches (102 mm)] wide, with 1/2 inch (13 mm) lip for poured concrete applications.

                - Attachment Method: [Cast-In, continuous extruded anchor] [Drilled Hole, Mechanically Fastened].

Short Tipped Nose: [2 inches (51 mm)] [3 inches (76 mm)] [4 inches (102 mm)] wide, with 1/4 inch (6 mm) lip for poured concrete and steel pan applications.

                - Attachment Method: Cast-In, continuous extruded anchor.

Long Nose Tread: [2-1/8 inches (54 mm)] [3-1/8 inches (79 mm)] [4 inches (102 mm)] wide, with 1-1/8 inch (31 mm) lip with 90 degree angle.

                - Attachment Method: [Cast-In, continuous extruded anchor] [Drilled Hole, Mechanically Fastened].

Inset, Square Front and Back: [2-1/4 inches (57.2 mm)]

                - Attachment Method: [Cast-In, continuous extruded anchor] [Drilled Hole, Mechanically Fastened].

Abrasive Colors: [Black] [Safety yellow] [see manufacturer for available color selections].

Finish: 6063-T5 aluminum, mill finish.

Specify taped top to prevent concrete from penetrating tread during installation. Painted undersides create a barrier between metal and concrete to avoid adverse reaction.

Options:

Taped Top.

Painted Undersides.

Renovation: Aluminum channel with 1 inch (25 mm) tapered back, abrasive filler consisting of aluminum oxide, silicon carbide, or a combination of both, in an epoxy-resin binder.

Basis-of-Design Product: Nystrom STRB

Nosing Types:

Long Nose Tread: [5 inches (127 mm)] [9 inches (229 mm)] [11 inches (280 mm)] wide, with 1-1/8 inch (31 mm) lip with 90 degree angle.

                - Attachment Method: Drilled Hole, Mechanically Fastened.

Angled Nose Tread: [5 inches (127 mm)] [9 inches (229 mm)] [11 inches (280 mm)] wide, with 1-1/8 inch (31 mm) lip with 83 degree angle.

                - Attachment Method: Drilled Hole, Mechanically Fastened.

Rounded (Bull) Nose Tread: 4 inches (102 mm) wide, with 1-3/16 inch (30.2 mm) lip.

                - Attachment Method: Drilled Hole, Mechanically Fastened.

Abrasive Colors: [Black] [Safety yellow] [see manufacturer for available color selections].

Finish: 6063-T5 aluminum, mill finish.

Specify taped top to prevent concrete from penetrating tread during installation. Painted undersides create a barrier between metal and concrete to avoid adverse reaction.

Options:

Taped Top.

Painted Undersides.

        - Two Part Extruded Units: One-part, Sub-channel and plywood insert; second part, Aluminum, with abrasive filler consisting of aluminum oxide, silicon carbide, or a combination of both, in an epoxy-resin binder.

Basis-of-Design Product: provide Nystrom [Ribbed Bar Abrasive, STTB] [Full Abrasive, STTF] with abrasive filler strips and aluminum extrusion.

Nosing Types:

Short Tipped Nose: 3-3/8 inches (86 mm) wide, with 1/2 inch (13 mm) lip for poured concrete and steel pan applications.

                - Attachment Method: [Cast-In, continuous extruded anchor].

Inset, Square Front and Back: 3-1/4 inches (83 mm) wide, with no lip, for poured concrete and steel pan applications.

                - Attachment Method: [Cast-In, continuous extruded anchor].

Abrasive Colors: [Black] [Safety yellow] [see manufacturer for available color selections]

Finish: 6063-T5 aluminum, mill finish.

Specify taped top to prevent concrete from penetrating tread during installation. Painted undersides create a barrier between metal and concrete to avoid adverse reaction.

Options:

Taped Top.

Painted Undersides.

      - Photoluminescent [abrasive] [translucent resin] metal [NOSINGS] [TREADS] [AND] [THRESHOLDS]

Retain product and nosing types required and revise to suit Project. Delete all if configurations are indicated on Drawings.

        - Extruded Units:  Aluminum with Photoluminescent [53 mil Resin Strip] [16 mil abrasive grit] and [abrasive filler consisting of aluminum oxide, silicon carbide][EcoTread], or a combination of both, in an epoxy-resin binder

Basis-of-Design Product: provide Nystrom STSM

Two-Part Short Tipped Ribbed Bar Abrasive Nosing 3-3/8 inch (86 mm) with 1 inch (25 mm) and 2 3/8 inch (60.3mm)  [abrasive filler][EcoTread], continuous extruded anchor attachment method.  Nystrom Model: STTM-P3.375E

Ribbed Abrasive Short Tipped Nose: 3 inches (76 mm) wide, 1 inch (25 mm) Photoluminescent [53 mil Resin Strip] [16 mil abrasive grit] and 2 inch (51 mm)  [abrasive filler][EcoTread]. Nystrom Model: STSM-P3E

                - Attachment method: [Cast-In, continuous extruded anchor] [Drilled Hole, Mechanically Fastened].

Inset, Square Front and Back: 3 inches (76 mm) with 1-inch (25 mm) Photoluminescent [53 mil Resin Strip] [16 mil abrasive grit] and  2 inch (51 mm)  [abrasive filler][EcoTread]. Nystrom Model: STSM-N3E

                - Attachment method: [Cast-In, continuous extruded anchor] [Drilled Hole, Mechanically Fastened].

Specify taped top or stair nosing guard to prevent concrete from penetrating tread during installation. Painted undersides create a barrier between metal and concrete to avoid adverse reaction.

Options:

Taped Top.

Painted Undersides

        - Extruded Units:  Aluminum with abrasive filler consisting of aluminum oxide, silicon carbide, or a combination of both, in an epoxy-resin binder.

Basis-of-Design Product: Subject to compliance with requirements, provide Nystrom; STSB, STTB

Retain types required and revise to suit Project. Delete all if configurations are indicated on Drawings

Ribbed Abrasive Short Tipped Nose: 3 inches (76 mm) wide, 1 inch (25 mm) 23 mil abrasive Photoluminescent grit and 2 inches (51 mm)  [abrasive filler][EcoTread]   .  Cast-in, continuous extruded anchor attachment method. Nystrom Model: STSB-P3E-V2FC1

Two-Part Short Tipped Nose Ribbed Bar Abrasive Nosing 3-3/8 inch (86 mm) with 1 inch (25 mm) 16 mil abrasive photoluminescent grit and 2 3/8 inch (60.3mm)  [abrasive filler][EcoTread]   . Cast-in, continuous extruded anchor attachment method.  Nystrom Model: STTB-P3.375E-V2FC1

Options

Taped Top.

Painted Undersides

        - Extruded Units for Renovation: Aluminum channel with 1 inch (25 mm) tapered back, Photoluminescent [53 mil Resin Strip] [16 mil abrasive grit].

Basis-of-Design Product: Subject to compliance with requirements, provide Nystrom; STRM-R2A

        - Photoluminescent Tapered Edge Translucent Resin Extrusion for Perimeter landing, exit path, door hardware, & door frame, Self-Adhesive 1 inch by 100 feet (25mm by 30.4 Meters).  Nystrom Model:  BEM-DS1-100.

      - ecotread RECYCLED RUBBER AND EXTRUDED ALUMINUM [NOSINGS] [TREADS] [AND] [THRESHOLDS]

        - Extruded Units: Aluminum, with EcoTread slip resistant, replaceable, recycled rubber.

- Basis of-Design Product: Nystrom, STSB-ECO, STRB-ECO, STTB-N-ECO

- Nosing Types:

- a.    Short Nose Tread: [1-7/8 inches (48 mm)] [3 inches (76 mm)] [4 inches (102 mm)] wide, with 1/2

- inch (13 mm) lip for poured concrete applications, with EcoTread recycled rubber.

- Nystrom Model: STSB-C1.875E-ECO, STSB-C3E-ECO, STSB-C4E-ECO

- 1)	Attachment Method: [Cast-In, continuous extruded anchor] [Drilled Hole, Mechanically Fastened].

- b.    Short Tipped Nose: [1-7/8 inches (48 mm)] [3 inches (76 mm)] [4 inches (102 mm)] wide, with

- 1/4 inch (6 mm) lip for poured concrete and steel pan applications, with EcoTread recycled rubber.

- Nystrom Model: STSB-P1.875E-ECO, STSB-P3E-ECO, STSB-P4E-ECO

- 1)	Attachment Method: [Cast-In, continuous extruded anchor] [Drilled Hole, Mechanically Fastened].

- c.    Long Nose Tread: [1-7/8 inches (48 mm)] [3 inches (76 mm)]  wide, with 1-1/8 inch (31 mm) lip with

- 90 degree angle, with EcoTread recycled rubber.

- Nystrom Model: STSB-L1.875E-ECO, STSB-L3E-ECO

- 1)	Attachment Method: [Cast-In, continuous extruded anchor] [Drilled Hole, Mechanically

- Fastened].

- d.    Angled Long Nose Tread: [1-7/8 inches (48 mm)] [3 inches (76 mm)] wide, with 1-1/8 inch (31 mm)

- lip, 83 degree angle for sloped risers, with EcoTread recycled rubber.

- Nystrom Model: STSB-A1.875E-ECO, STSB-A3E-ECO

- 1)	Attachment Method: [Cast-In, continuous extruded anchor] [Drilled Hole, Mechanically Fastened].

- e.    Inset, Square Front and Back: [1-3/8 inches (35 mm)] [3 inches (76 mm)] with EcoTread recycled

- rubber.  Nystrom Model: STSB-N1.375E-ECO, STSB-N3E-ECO

Attachment Method: [Cast-In, continuous extruded anchor] [Drilled Hole, Mechanically Fastened]

- f.    Renovation Long Nose Tread: [5 inches (127 mm)] [9 inches (229 mm)] [11 inches (280 mm)]

- wide, with 1-1/8 inch (31 mm) lip with 90 degree angle, with EcoTread recycled rubber.

- Nystrom Model: STRB-L5D-ECO, STRB-L9D-ECO, STRB-L11D-ECO

- 1)	Attachment Method: Drilled Hole, Mechanically Fastened.

- g.    Renovation Angled Nose Tread: [5 inches (127 mm)] [9 inches (229 mm)] [11 inches (280 mm)]

- wide, with 1-1/8 inch (31 mm) lip with 83 degree angle, with EcoTread recycled rubber.

- Nystrom Model: STRB-A5D-ECO, STRB-A9D-ECO, STRB-A11D-ECO

- Attachment Method: Drilled Hole, Mechanically Fastened.

- h.    Inset, Square Front and Back: 3-1/4 inches (83 mm) wide, with no lip, for poured concrete and

- steel pan applications.  Nystrom Model: STTB-N3.25E-ECO

- Attachment Method: [Cast-In, continuous extruded anchor].

Finish: 6063-T5 aluminum, mill finish.

EcoTread Colors: [Black] [Safety Yellow] [Black/Blue] [Black/Gray].

Specify taped top or stair nosing guard to prevent concrete from penetrating tread during installation. Painted

undersides create a barrier between metal and concrete to avoid adverse reaction.

Options

Taped Top.

Painted Undersides

Extruded Units: Aluminum, with EcoTread slip resistant, replaceable, recycled rubber combined with Photoluminescent insert.

- Basis of Design Product: Nystrom, STSM-ECO, STTM-ECO, STRM-ECO

- Nosing Types:

Ribbed Abrasive Short Tipped Nose: 3 inches (76 mm) wide EcoTread, 1 inch (25 mm) Photoluminescent [53 mil Resin Strip] [21 mil abrasive grit] and 2 inches (51 mm) EcoTread.  Nystrom Model STSM-P3E-ECO, STSM-P3E-T-ECO

Attachment method: [Cast-In, continuous extruded anchor] [Drilled Hole, Mechanically Fastened].

Inset, Square Front and Back: 3 inches (76 mm), 1-inch (25 mm) Photoluminescent [53 mil Resin Strip] [16 mil abrasive grit] and 2 inches (51 mm) EcoTread.

- Nystrom Model: STSM-N3E-ECO, STSM-N3E-T-ECO

Attachment Method: [Cast-In, continuous extruded anchor] [Drilled Hole, Mechanically Fastened].

- c.	Two-Part Short Tipped Nose Ribbed Bar Abrasive Nosing 3-3/8 inch (86 mm) wide EcoTread, 1 inch (25 mm) Photoluminescent [53 mil Resin Strip] [21 mil abrasive grit] and 2 3/8 inch (60.3mm)  EcoTread.  Nystrom Model: STTM-P3.375E-ECO, STTM-P3.375E-T-ECO

- Attachment method: [Cast- In, continuous extruded anchor].

- d.      Renovation Long Nose Tread: [5 inches (127 mm)] wide EcoTread, with 1-1/8 inch (31 mm) lip with

- 90 degree angle.  Photoluminescent [53 mil Resin Strip] [21 mil abrasive grit] and 4 inches (102

- mm) EcoTread.  Nystrom Model STRM-L5D-ECO, STRM-L5D-T-ECO

- 1)	Attachment Method: Drilled Hole, Mechanically Fastened.

- e.     Renovation Angled Nose Tread: [5 inches (127 mm)] wide, EcoTread, with 1-1/8 inch (31 mm) lip

- with 83 degree angle.  Photoluminescent [53 mil Resin Strip] [21 mil abrasive grit] and 4 inches

- (102 mm) EcoTread.  Nystrom Model STRM-A5D-ECO, STRM-A5D-T-ECO

- 1)	Attachment Method: Drilled Hole, Mechanically Fastened.

Finish: 6063-T5 aluminum, mill finish.

EcoTread Colors: [Black] [Safety Yellow] [Black/Blue] [Black/Gray].

Specify taped top or stair nosing guard to prevent concrete from penetrating tread during installation. Painted undersides create a barrier between metal and concrete to avoid adverse reaction.

Options:

Taped Top.

Painted Undersides

      - Cast-Metal Units: Cast aluminum, with an integral-abrasive, as-cast finish consisting of aluminum oxide, silicon carbide, or a combination of both.

Basis-of-Design Product: provide Nystrom; STCA

Provide cross-hatched units with silicon carbide abrasive 20 grit, a minimum of 2-1/4 ounces/sq. ft. (687 grams/sq. m), to a minimum of 1/32 inch (0.8 mm) thick.

Retain types required and revise to suit Project. Delete all three if configurations are indicated on Drawings.

Nosing Type:

Short Nose: Cross-hatched units, [3 inches (76 mm)] wide with 1/4-inch (6-mm) lip.

Attachment Method: [Anchor-In-Place]  [Wing anchor].

Short Tipped Nose: Cross-hatched units, [3 inches (76 mm)].

Attachment Method: [Anchor-In-Place]  [Wing anchor].

Specify taped top to prevent concrete from penetrating tread during installation. Painted undersides create a barrier between metal and concrete to avoid adverse reaction.

Options:

Taped Top.

Painted Undersides.

EXECUTION

      - EXAMINATION

        - Verify that preparation and affected dimensions are acceptable.

        - Verify tolerances and correct improper conditions.

      - PREPARATION

        - Advise installers of details relating to floor hatch installation, including rough opening dimensions, locations of supports, and anchoring methods.

      - INSTALLATION

        - Follow manufacturer’s instructions for installing floor doors and hatches.

        - Install frames plumb and level in opening, in proper alignment with floor surface for flush installation.  Secure rigidly in place.

        - Position units to provide convenient access to concealed Work requiring access.

      - ADJUSTING

        - Operational Units:  Test-operate units with operable components.

        - Clean and lubricate joints and hardware.

        - Adjust for proper operation.

      - CLEANing

        - Clean adjacent surfaces and remove unused product and debris from site.

        - Adjust doors for smooth operation.

END OF SECTION